Title: Life cycle assessment of passenger cars fed with a blend of LPG, bio-LGP, and renewable DME, and comparison with conventional and electric passenger cars

Author(s): Stefano Puricelli*1, Michelle Eid1, Simone Casadei2, Mario Grosso1, Giovanni Lonati1, Lucia Rigamonti1, Stefano Cernuschi1

1 Department of Civil and Environmental Engineering, Politecnico di Milano, Milano, Italy, stefano.puricelli@polimi.it, stefano.cernuschi@polimi.it, michelle.eid@polimi.it, mario.grosso@polimi.it, giovanni.lonati@polimi.it, lucia.rigamonti@polimi.it

2 Sustainable Mobility Team, Innovhub – Stazioni Sperimentali per l’Industria, Milano, Italy, simone.casadei@mi.camcom.it

Keyword(s): LCA, transport, biofuels, e-fuels, LPG, DME, electric car, petrol

Abstract

The Life Cycle Assessment (LCA) methodology was applied to comparatively assess the environmental impacts of passenger cars (from both segments B and C) with spark-ignition (SI) engine and with electric motor. The SI cars complied with the Euro 6d emission standard and were fuelled with petrol or a mixture of liquefied petroleum gas (LPG) (60% m/m), bio-LPG (20% m/m), and renewable dimethyl ether (20% m/m). The Life Cycle Assessment (LCA) methodology was applied for evaluating comparatively the environmental impacts of two passenger cars (of B and C segments) powered by:

- a SI engine fuelled by petrol;

- a SI engine fuelled by a mixture of LPG (60% m/m), bio-LPG (20% m/m), and renewable dimethyl ether (20% m/m);

- an electric motor.

Three types of dimethyl ether (DME) were tested in the evaluation: the biofuel (bio-DME), the e-fuel (e-DME), and the recycled carbon fuel (RC-DME). The fuels for SI engine cars were analysed and tested by Innovhub-SSI on a chassis dynamometer laboratory test bench, using two B-segment cars and one C-segment car. The results for the two B-segment cars were averaged. All the passenger cars belonged to the category of Sport Utility Vehicles (SUV). Moreover, two battery electric cars, those modelled in [1], were considered. The functional unit was defined as “traveling 1 km in Italy with a passenger car”. The total mileage covered by the cars was set at 190,000 km for the B segment and 210,000 km for the C segment [2]. The system boundaries include the entire life cycle of the car (production, use, maintenance, end of life) and the energy carriers that power it, as well as the construction and end of life of the road. Multifunctionality issues in the foreground system were solved following the hierarchy recommended by [3], although in some cases it was preferred to deviate from it. For instance, for modelling processes involving waste treatment activities or the use of waste streams as feedstock, the “polluter pays” approach was adopted: impacts occurring until the waste has a negative market price are attributed to the system that generated the waste. The geographical horizon of the LCA is Italy and the time horizon is the year 2023. The modelling of the foreground system was primarily based on secondary data from literature or specific websites, while for the background system it was based on secondary data acquired from the ecoinvent database version 3.9.1 (cut-off system model) [4]. The analysis was conducted using SimaPro 9.5 software. The evaluation considered the 16 impact categories suggested in [5]. Carbon neutrality was assumed for CO2 emissions from the biofuel combustion. CO2 captured from a cement plant, incorporated into a renewable fuel and subsequently combusted, was also considered neutral.

It was assumed that petrol production takes place in Italy, as it accounts for 97% of the petrol consumed locally [6], [7]. To model the production chain of LPG, a questionnaire was prepared and sent to four Italian LPG-selling companies, for a total of 508,000 t sold in 2022. Based on the information gathered from the questionnaires, 27% of LPG is produced in Italy and 73% is imported. Bio-LPG (also referred to as biopropane) was modelled as a by-product of the hydrotreatment process called Hydroprocessed Esters and Fatty Acids (HEFA), yielding HEFA-diesel as primary product. The assessed scenario refers to the future configuration of the “Porto Marghera” biorefinery (Venice, Italy), detailed in the IPPC permit documentation [8]–[11]. In this analysis, the most used raw material in Italy for HEFA production, namely used cooking oil (UCO), was considered. According to 2018 data [12], the UCO used in the European Union (EU) to produce conventional biodiesel (FAME) and HEFA-diesel comes for 42% from EU countries, 21% from China, 11% from Indonesia, and the remaining 26% from other non-European countries. After a series of pretreatments for reducing the contaminants inside the UCO, this feedstock is treated with hydrogen (produced from natural gas) and the final products of the process are HEFA-diesel, bionaphtha, bio-jet fuel, and bio-LPG. The production of bio-DME was modelled according to [13] and involves two stages: the first one is the dry reforming of methane and CO2 (captured) into syngas, the second one is the direct synthesis of DME. The CO2 capture process was added to the model of [13] by using data retrieved in [14]. For this study, fossil methane was replaced with its biogenic counterpart, biomethane, which comes from the organic fraction of municipal solid waste and is the most used raw material in Italy to produce biomethane for automotive use [15]. To model the production of e-DME, data retrieved in [16] were used. In this theoretical model, methanol is firstly produced by CO2 hydrogenation, with CO2 captured from the exhaust gases of a cement plant; e-DME is then obtained through methanol dehydration. The electricity required for electrolysis was modelled using the Italian hydropower production mix [17], as this was the highest consumed one (39.1%) among renewable sources used in Italy in 2021 [18]. The production process of RC-DME was modelled by using data from a study on methanol production from waste plastics (50% polypropylene and 50% polyethylene) [19]. The plastics, after being transported to the plant, are crushed to 5 mm and fed into the gasification reactor. The resulting syngas undergoes a series of purification stages and is then subjected to steam reforming. Methanol with a purity of 99.8% is synthesised from the resulting syngas. The final conversion into DME was based on data from [16].

Exhaust emissions and fuel consumptions from the SI cars were obtained from measurements conducted in the laboratory of Innovhub-SSI. Driving tests were conducted according to the WLTC (Worldwide harmonized Light vehicle Test Cycle). Measurements included regulated pollutants (CO, total hydrocarbons [THC], non-methane hydrocarbons [NMHC], NOx, particulate matter [PM], and solid particle number [PN]) as well as other conventional and trace compounds of significance for traffic environmental evaluations (CO2, CH4, PM soot, total PN (via EEPS), NH3, N2O, 1,3-butadiene, formaldehyde, and benzene). To complete the exhaust emissions inventory of SI cars with evaporative emissions and other trace compounds unmeasured in laboratory and coming from the combustion of fuels and lubricant oil, data available in [20] were used. Electric car consumptions were calculated by a weighted mean of WLTC consumptions of the B- and C-segment SUVs models mostly registered in Italy in the first nine months of 2023 [21], [22]. Production and distribution of electricity refer to the Italian electricity mix of 2019. The demand for materials for the domestic charging column of the electric car was also included in the inventory [23]. Emissions due to tyre, brake, and road surface wear were estimated using emission factors available in [20]. For electric cars, emissions from brake wear were decreased by correcting the corresponding SI cars’ emission factors with specific scaling coefficients derived from [20]. This is because of the presence of regenerative braking, which significantly reduces the use of conventional braking systems. Evaporative emissions were estimated only for the petrol cars, for which an emission factor was available in [20]. The production, maintenance, and end-of-life phases of the vehicles’ life cycle were modelled based on the datasets retrieved from [1].

Table 1 shows the percentage differences in environmental impacts compared to the petrol car. Red and bold digits highlight variations greater than +10%, whilst green and italic digits highlight variations greater than -10%. Due to the inherent uncertainty in every LCA, variations below 10 percentage points were not considered significant (black digits). The type of renewable DME did not significantly influence the results, due to its low content in the blend (20% by mass), except for the water use impact category. For this reason, it was preferred to calculate and report in Table 1 an average of the results of the three rDME blends.

Table 1. Percentage differences in impacts compared to the petrol car scenario.

| Impact category | SI engine with LPG + bio-LPG + rDME* (B segment) | Electric motor (B segment) | SI engine with LPG + bio-LPG + rDME* (C segment) | Electric motor (C segment) |
| --- | --- | --- | --- | --- |
| Acidification | +5% | +55% | +10% | +50% |
| Climate change | -21% | -36% | -16% | -38% |
| Ecotoxicity, freshwater | -36% | -27% | -34% | -29% |
| Particulate matter | -5% | -1% | -3% | -4% |
| Eutrophication, marine | +8% | +45% | +17% | +39% |
| Eutrophication, freshwater | +13% | +122% | +14% | +113% |
| Eutrophication, terrestrial | +8% | +31% | +19% | +26% |
| Human toxicity, cancer | +1% | +44% | +3% | +40% |
| Human toxicity, non-cancer | +2% | +141% | +3% | +135% |
| Ionising radiation | +8% | +170% | +9% | +155% |
| Land use | -0.5% | +31% | +0.4% | +25% |
| Ozone depletion | -27% | -32% | -24% | -35% |
| Photochemical ozone formation | -33% | -37% | -23% | -33% |
| Resource use, fossils | -18% | -31% | -13% | -33% |
| Resource use, minerals and metals | +7% | +134% | +8% | +120% |
| Water use | +372% | +236% | +394% | +219% |

* The results of the sub-scenarios involving the use of the three different types of rDME have been averaged.

As easily predictable, an increase in absolute impacts is observed across all categories when transitioning from the B-segment to the C-segment car. The use of the innovative blends shows improvement over petrol in five impact categories, including climate change, for both segments, with reductions ranging from 13% to 36%. The use of electric cars, for both segments, also shows improvement over petrol in five impact categories but deterioration in ten, especially for water use, ionising radiation, non-carcinogenic human toxicity, and freshwater eutrophication. All the alternatives, when compared to petrol, require a relevant increase in water use. Comparing the estimated percentage differences for SI and electric cars, the former would offer better environmental performance in 11 of the 16 impact categories for the B-segment and in 10 out of 16 for the C-segment, but not for the particularly significant one of climate change. More specifically, for this impact category, the innovative blend of GPL, bio-GPL, and rDME reduces impacts on average by 21% in B-segment cars and 16% in C-segment car, whilst the electric car lowers impacts by 36% in B-segment cars and 38% in C-segment car. The impact on climate change is primarily due to fossil CO2 and CH4 emissions from petrol and LPG combustion, and to the activities related to the production of energy carriers and cars. In the case of petrol cars, more than half of the impact is due to exhaust emissions, with the corresponding reduction resulting for the innovative blend originated by the inclusion of biofuels. The electric car emerges as the best option for climate change because the absence of exhaust emissions is more than capable to offset the higher impacts associated with car production and with battery recharging.

References

[1] Sacchi R., Bauer C., “Life-cycle inventories for on-road vehicles”, (2023). https://doi.org/10.5281/zenodo.5156043

[2] Weymar E., Finkbeiner M., “Statistical analysis of empirical lifetime mileage data for automotive LCA”, Int. J. Life Cycle Assess., vol. 21, no. 2, pp. 215–223, (2016).

[3] ISO, “ISO 14044:2006/Amd 2:2020 Environmental management - Life cycle assessment - Requirements and guidelines”, (2020). https://www.iso.org/standard/76122.html

[4] Wernet G., Bauer C., Steubing C., Reinhard J., Moreno-Ruiz E., Weidema B., “The ecoinvent database version 3 (part I): overview and methodology”, Int. J. Life Cycle Assess., vol. 21, no. 9, pp. 1218–1230, (2016). doi: 10.1007/s11367-016-1087-8.

[5] European Commission, “Commission Recommendation (EU) 2021/2279”, Off. J. Eur. Union, vol. 471, (2021).

[6] Unem, “Produzione delle raffinerie (volumi)”, (2021). https://www.unem.it/i-numeri-dellenergia/italia/

[7] Unem, “Importazioni di benzine, gasoli e olio combustibile per Paesi di provenienza”, (2022). https://www.unem.it/i-numeri-dellenergia/italia/

[8] Eni S.p.A., “Raffineria di Venezia - Dichiarazione Ambientale 2020-2022 - Edizione 2021”, (2021). https://www.eni.com/assets/documents/ita/attivita/mid-downstream/Dichiarazione-Ambientale-Venezia-2021.pdf

[9] Eni S.p.A., “Progetto ‘Steam Reforming’”, (2022). https://va.mite.gov.it/it-IT/Oggetti/Documentazione/8802/12947

[10] Eni S.p.A., “Riesame AIA - Scheda A: Allegati”, (2020). https://va.minambiente.it/en-GB

[11] Eni S.p.A., “Riesame AIA - Scheda B: Dati e notizie sull’impianto attuale”, (2019). https://va.minambiente.it/en-GB

[12] European Commission, “COM(2020) 952 final”, (2020). https://eur-lex.europa.eu/legal-content/GA/TXT/?uri=CELEX:52020DC0952

[13] Fernández-Dacosta C., Shen L., Schakel W., Ramirez A., Kramer G.J., “Potential and challenges of low-carbon energy options: Comparative assessment of alternative fuels for the transport sector”, Appl. Energy, vol. 236, pp. 590–606, (2019). doi: 10.1016/j.apenergy.2018.11.055.

[14] Meunier N., Chauvy R., Mouhoubi S., Thomas D., De Weireld G., “Alternative production of methanol from industrial CO2”, Renew. Energy, vol. 146, pp. 1192–1203 (2020). doi: 10.1016/j.renene.2019.07.010.

[15] GSE S.p.A., “Energia nel settore dei trasporti 2005-2021”, (2022).  https://www.gse.it/documenti_site/Documenti GSE/Rapporti statistici/Energia nel settore Trasporti 2005-2021.pdf

[16] Galusnyak S.C., Petrescu L., Chisalita D.A., Cormos C-C., “Life cycle assessment of methanol production and conversion into various chemical intermediates and products”, Energy, vol. 259, p. 124784, (2022).

[17] Terna S.p.A., “Produzione 2022”, (2023). https://www.terna.it/it/sistema-elettrico/statistiche/pubblicazioni-statistiche

[18] Terna S.p.A., “Dati statistici sull’energia elettrica in Italia - 2021”, (2022). https://www.terna.it/it/sistema-elettrico/statistiche/pubblicazioni-statistiche

[19] Afzal S., Singh A., Nicholson S.R., Uekert T., DesVeaux J.S., Tan E.C.D., Dutta A., Carpenter A.C., Baldwin R.M., Beckham G.T., “Techno-economic analysis and life cycle assessment of mixed plastic waste gasification for production of methanol and hydrogen”, Green Chem., vol. 25, no. 13, pp. 5068–5085, (2023).

[20] EEA, “EMEP/EEA air pollutant emission inventory guidebook 2023”, (2023). https://www.eea.europa.eu/publications/emep-eea-guidebook-2023

[21] UNRAE, “Immatricolazioni autovetture e fuoristrada BEV - 9 mesi 2023”, (2023). https://unrae.it/dati-statistici/immatricolazioni/6436/immatricolazioni-bev-per-modello-settembre-2023

[22] Quattroruote, “Listino Nuovo”, (2023). https://www.quattroruote.it/listino/

[23] Lucas A., Alexandra Silva C., Costa Neto R., “Life cycle analysis of energy supply infrastructure for conventional and electric vehicles”, Energy Policy, vol. 41, pp. 537–547, (2012).