PDADMA-(TFSI) Membranes from DMF Solutions: Synthesis and Comprehensive Mechanical & Transport Property Study

Soroush Sabbaghi1,2*, Giacomo Melani2, Alessandro Mariani1, Silvia Leonardi2, Guido Raos1

1 Dept. of Chemistry, Materials and Chemical Engineering "G. Natta", Politecnico di Milano, Via L. Mancinelli 7, 20131 Milano, Italy

2 Renewable, New Energies & Material Science Research Center (DE-R&D), Eni S.p.A, Via Giacomo Fauser 4, 28100, Novara, Italy

*email: soroush.sabbaghi@polimi.it

Organic redox flow batteries (ORFBs) present a promising avenue for sustainable energy storage due to their superior charge density compared to conventional redox flow batteries [1]. Their capabilities position them as strong candidates for large-scale energy storage solutions. However, this emergent field requires significant effort to optimize performances. In this study, we mainly utilize classical molecular dynamics (MD) simulations to elucidate and quantify the properties of potential ORFB candidates. We focus on the nano-structural, mechanical, and transport characteristics of membranes in various electrolyte solutions, as well as their interfacial interactions.

Our investigation focuses on the use of Dimethylformamide (DMF) as a solvent for the membrane casting and Polydiallyldimethylammonium (PDADMA) polymer combined with trifluoromethanesulfonyl imide (TFSI) anion to neutralize charges [2]. We begin by simulating pure DMF to identify the optimal potential and partial charges necessary for our simulation purposes [3]. Following this, we simulate a PDADMA-TFSI system solvated in DMF. These simulations offer valuable insights into the influence of solvent selection and synthesis methodology on the final nanostructure of the membrane, as well as strategies for tuning the mechanical and transport properties accordingly.

The majority of the simulations conducted in this research utilized the Large-scale Atomic/Molecular Massively Parallel Simulator (LAMMPS), implementing the non-polarizable OPLS-AA force field for both molecular solvents and polymers [4]. In future work, Through the evaporation of DMF, we predict the nanostructure of the resulting membrane.

References

[1]	S. Ahn, A. Yun, D. Ko, V. Singh, J. M. Joo, and H. R. Byon, “Organic redox flow batteries in non-aqueous electrolyte solutions,” Chem Soc Rev, 2024, doi: 10.1039/d4cs00585f.

[2]	W. Jaeger, J. Bohrisch, and A. Laschewsky, “Synthetic polymers with quaternary nitrogen atoms-Synthesis and structure of the most used type of cationic polyelectrolytes,” Progress in Polymer Science (Oxford), vol. 35, no. 5, pp. 511–577, 2010, doi: 10.1016/j.progpolymsci.2010.01.002.

[3]	V. Vasudevan and S. H. Mushrif, “Force field parameters for N,N-Dimethylformamide (DMF) revisited: Improved prediction of bulk properties and complete miscibility in water,” J Mol Liq, vol. 206, pp. 338–342, 2015, doi: 10.1016/j.molliq.2015.03.004.

[4]	W. L. Jorgensen and J. Tirado-rives, “The OPLS [optimized potentials for liquid simulations] potential functions for proteins, energy minimizations for crystals of cyclic peptides and crambin},” J Am Chem Soc, no. 4, 1988, doi: https://doi.org/10.1021/ja00214a001.